DEVELOPMENT PLAN (From Prototype to Final System)

PHASE 1: Hardware

We need to know if the board is alive or not by do a checklist:

* Power the board via USB.

* Measure VBUS (5V) from laptop or any source, and 3.3V rail.

* Finally, check the board is overheating or not, and there is also no short circuit.

For process:

- Plug in USB (no firmware yet)

- Measure VBUS = 5V

- Measure 3.3V output

- Check current consumption (<100mA)

=> If anything is wrong happen, debug the power tree one more times.

PHASE 2: MCU

In this phase, we need our MCU-STM32 chip is running.

The checklist:

* Program via STM32CubeIDE 2.0.0.

* Blink LED.

If fail:

* Check clock

* Check BOOT0

* Check SWD connection

PHASE 3: LCD + KEYPAD

Make sure the LCD and KEYPAD is working when in and out the data

For process:

- Test the LCD: By display any world, ex: CAT

- Test the KEYPAD: Press key and print it

- Combine two of them: When we press KEYPAD, it displays on LCD

PHASE 4: ADC (for analog input) (optional)

We need stable analog reading

In here we need to check:

+ If the signal noisy?

+ If yes: - Verify VDDA filtering

- Add decoupling capacitors

PHASE 5: USB CDC

We need our PC (or LAPTOP) detect our caculator

So that we need to check: when we plug USB, will it appears in Device Manager as “Virtual COM Port”?

For the setup, we also need program via STM32CubeIDE 2.0.0 to enable USB device for CDC class

PHASE 6: Display the data on Web interface.

In this case, we will use Web Serial API for simple process.

For the steps: STM32 → USB CDC → Browser → Web App

PHASE 7: System Integration

Now the caculator has to be function. And the caculator will have to work with the flow:

Keypad → STM32 → LCD

│-> ADC → analog value

│-> USB → Web display

PHASE 8: Testing & Debugging

Checklist:

* Keypad responsiveness

* USB stability (disconnect issues?)

* ADC noise level

* LCD flickering

PHASE 9: Optimization

* Reduce ADC noise (better grounding, filtering)

* PWM control for LCD backlight

* Implement low-power modes

FINAL PHASE: Documentation

It will have to include in our report:

* Block diagram

* Power tree

* Schematic

* Firmware flowchart

* Testing results